F4-424 Pro BIOS Update Guide

- 1.Format USB flash disk as FAT32

- Extract the downloaded ZIP file and copy all extracted files directly to the root directory of the USB drive, as shown in the image below.

F4-424 / F4-424 Pro

- F4-424 Max

- Power off the NAS machine that needs a BIOS upgrade, remove the SATA drive, M.2 NVME SSD, and TOS initboot USB drive, and remove the keyboard and mouse. Then insert the BIOS upgrade USB drive.

- Plug in the power and turn on the NAS. The HDMI monitor will automatically display the BIOS update information. Once you see the content shown in the image below, it means you have entered the BIOS update environment and are about to begin updating the BIOS.

5，The BIOS update will be completed automatically. Once you see the message shown in the image below, the BIOS update is finished. Then, shut down the NAS and remove the USB drive.

- Please check the BIOS Version and Build Date and Time information in the BIOS Setup page. If the displayed values match the versions listed below, the BIOS upgrade has been completed successfully.

F4-424 / F4-424 Pro

- BIOS Version: BJFX-ADLN-109

- Build Date and Time: 12/15/2025 16:14:28

- F4-424 Max

- BIOS Version: T1510X25

- Build Date and Time: 01/17/2025 14:34:21

- If both the BIOS Version and Build Date and Time match the information above, it indicates that the BIOS has been successfully upgraded to the latest version.